APT REPORT ON

EVALUATION OF SPECTRUM UTILISATION IN SATRC MEMBER COUNTRIES

Edition: October 2023

The 24th Meeting of South Asian Telecommunication Regulators’ Council (SATRC-24)

3 – 5 October 2023

Dhaka, Bangladesh

(Source: SATRC-24/OUT-06)

No. SAPVIII-REP-06

Adopted by

24th Meeting of South Asian Telecommunication Regulators’ Council (SATRC-24)

3 - 5 October 2023

Dhaka, Bangladesh

(Source: SATRC-24/OUT-06)

Content

1-	Chapter 1: Introduction	4

2-	Chapter 2: General description of terrestrial Mobile System	5

2-1-	Introduction	5

2-2-	Cellular terrestrial mobile standards	5

2-1-1-	P25	5

2-1-2-	TETRA	6

2-1-3-	DMR	6

2-1-4-	dPMR	6

2-1-5-	IMT	6

3-	Chapter 3: Spectrum utilization	11

3-1-	Introduction	11

3-2-	Spectrum utilization theory	11

3-3-	Spectrum utilization measurement	12

3-3-1-	Spectrum measurement	12

3-3-2-	Evaluate of the Spectrum Utilization	13

4-	Chapter 4:  Spectrum Utilization Efficiency	16

4-1-	Introduction	16

4-2-	Theory of Spectrum utilization efficiency (SUE)	16

4-3-	Evaluate of SUE in SATRC’s countries based on received answers	16

4-4-	Evaluate of SUE in IMT networks based on Iran’s experience	18

4-4-1-	2G Network	19

4-4-2-	3G Network	19

4-4-3-	4G Network	19

5-	Chapter 5: Evaluate of relative spectrum utilization efficiency (RSE)	20

5-1-	Theory of RSE	20

5-2-	Evaluate of standard SUE in SATRC’s countries based on received answers.	20

5-3-	Evaluate of standard SUE in IMT networks based on Iran’s experience	21

5-3-1-	2G Network	21

5-3-2-	3G Network	21

5-3-3-	4G Network	22

6-	Chapter 6: Spectrum audit	23

6-1-	Background	23

6-2-	Audit Objectives	23

6-3-	Audit Methodology	24

6-4-	Sources of Audit Criteria	24

6-5-	Audit methods	25

6-5-1-	ITU	25

6-5-2-	CEPT	25

6-5-3-	FCC	25

6-5-4-	SATRC countries	25

6-5-5-	Iran’s experience	26

7-	Chapter 7: Conclusion	27

7-1-	Aim of evaluating spectrum utilization	27

7-2-	Audit	27

7-2-1-	Audit Observations	27

7-2-2-	Key Audit Recommendations	28

7-2-3-	Benefits of Spectrum Audit	28

8-	Chapter 8: Questions and Answers	29

8-1-	Questions	29

8-2-	Answers	30

8-2-1-	Pakistan	30

8-2-2-	Sri Lanka	32

8-2-3-	Bangladesh	33

8-2-4-	Nepal	34

8-2-5-	India	35

8-2-6-	Iran	36

8-2-7-	Bhutan	37

8-2-8-	Afghanistan	38

8-2-9-	Maldives	39

# Chapter 1: Introduction

‘Spectrum’ is collection of various types of electromagnetic radiations of different wavelengths. Spectrum is a finite and scarce but non-exhaustible natural resource which is a vital input for wireless services. With its broad range of applications in personal and business communications, it has ever increasing economic and social utility. The consumer and business benefits derived from it are very significant.  Rapid development of wireless technology has ensured that there is an increasing range of valuable uses of the spectrum. With competing users, uses and growth of wireless services, the demand for spectrum has tremendously increased. The trend of modern communications is towards mobility, with increasingly higher data rates/ speeds, for which wireless is the only option. The requirements of captive applications are also growing. All these have resulted in greater demands/ pressure on the already scarce RF spectrum resource.

Because there is a finite amount of spectrum and a growing demand for it, effectively managing the available spectrum is a strategic issue for the Spectrum Regulators. Increasing demand for spectrum-based services and devices is straining long standing spectrum policies. There is a growing need to appropriately change to exploit the full potential of spectrum use through market orientation and stimulating development of new technologies that are more efficient and economize the spectrum usage, because future success and growth of telecommunications sector depends heavily on effective and efficient spectrum management. Unless spectrum can be made available to meet the ever growing demand, there will be a setback to innovations and competition and businesses as well as consumers will be worse off. Making spectrum available at a time when convergence is causing rapid and unpredictable change poses a severe challenge. Advances in technology create the potential for systems to use spectrum more efficiently and to be much more tolerant of interference than in the past.

As the wireless industry is witnessing explosive growth in the demand for both voice and data services, the number of mobile telephone subscribers, as well as usage rates, have also grown considerably. Consequently, the Service providers have been upgrading their networks with advanced technologies to meet this growing demand for high quality voice and data services. The equipment vendors are driving technical innovations with the latest wireless technologies showing significant gains in the efficiency of spectrum used, thus providing more capacity out of a given bandwidth.

The bandwidth intensive services that users will want, and the rising number of users, are placing increasing demands on access networks. These demands may eventually not be met by the enhancement of radio access systems (in terms of peak bit rate to a user, aggregate throughput, and greater flexibility to support many different types of service simultaneously). Therefore there will be a requirement for new radio access technologies to satisfy the anticipated demands for higher bandwidth services. As third generation (3G) International Mobile Telecommunication 2000 (IMT-2000) systems are being deployed, further developments aiming at their enhancement are being conducted on a worldwide scale.  Many operators in the developed countries are focusing on deploying IMT – advanced system networks to cater to the growing requirement of data, speed and content delivery.

# Chapter 2: General description of terrestrial Mobile System

## Introduction

A Terrestrial Mobile System is a person-to-person voice communication system consisting of two-way radio transceivers (an audio transmitter and receiver in one unit) which can be stationary (base station units), mobile (installed in vehicles), or portable (handheld walkie-talkies). Public land mobile radio systems are made for use exclusively by public safety organizations such as police, fire, and ambulance services, and other governmental organizations, and use special frequencies reserved for these services. Private land mobile radio systems are designed for private commercial use, by firms such as taxis or delivery services.

Cellular telephony derives its name from the partition of a geographic area into small “cells”. Each cell is covered by a local radio transmitter and receiver just powerful enough to enable connectivity with cellular phones, referred to also as mobile terminals, within its area (Figure 1). The set of cells forms the radio access network, and the radio frequencies used for the transmission of calls and data can be reused between cells. A different type of reuse, digital code reuse, Voice and data exchanged between a mobile terminal and regular phone networks, or the Internet, are transmitted via the mobile network which consists of the cellular operator’s radio access network and core network.

Figure 1: Mobile network overview

## Cellular terrestrial mobile standards

### P25

Project 25 (P25) is a suite of standards for digital radio communications published by The Association of Public Safety Communications Officials (APCO).  The specification was introduced in October 1989.  P25 includes standards for the design and manufacture of interoperable digital two-way wireless communications products intended for public safety professionals.

### TETRA

Terrestrial Trunked Radio (TETRA), formerly known as Trans-European Trunked Radio, is an open standard developed by the European Telecommunications Standards Institute (ETSI) and was first published in 1995.  TETRA was specifically designed for use by government agencies, emergency services, police forces, fire departments, paramedics, and emergency medical technicians for public safety networks.  In addition to public safety, it is used in many commercial applications where voice and data communications services are required such as transportation services, utilities, and even the military.

### DMR

Digital Mobile Radio (DMR) is a digital radio standard developed by the European Telecommunications Standards Institute (ETSI) and was first ratified in 2005.  The standard is designed to operate within the existing 12.5 kHz channel spacing used in licensed land mobile frequency bands globally and to meet future regulatory requirements for 6.25 kHz channel equivalence.  The primary goal is to specify affordable digital systems with low complexity.  DMR provides voice, data and other supplementary services.  Products designed to its specifications are sold in all regions of the world.  The DMR protocol covers unlicensed (Tier I), licensed conventional (Tier II) and licensed trunked (Tier III) modes of operation.  Commercial application is focused on the Tier II and III licensed categories.

### dPMR

Digital Private Mobile Radio (dPMR) was developed by the European Telecommunications Standards Institute (ETSI) and published under the reference ETSI TS 102 658.  Although dPMR and DMR were all launched around the same time, DMR has seen the greatest success, due to the lower infrastructure costs (two voice paths for one base station).   dPMR manufacturers have considerably cut infrastructure costs such that for small scale systems dPMR is more cost effective to deploy.

### IMT

#### Overview

The initiation of IMT Technology was due to the ‘globalization’ trend in all forms of communications, business, or even entertainment. Globalization is necessary in order to have sufficient flexibility to meet local needs and to allow regional or national systems to evolve smoothly toward international involvement for wireless communications.

Thus, the International Telecommunication Union (ITU) has the objective to develop a flexible standard and universal telecommunications infrastructure especially for wireless access. The standard should serve mobile and fixed users in both public and private networks. The ITU began its studies on global telecommunications in 1986, when the availability of handheld cellular phones offered the potential for worldwide usage, rather than just national, regional or only land mobile systems.   Based on this globalization vision, the ITU World Administrative Radio Conference in 1992 (WARC‐92) identified 1885‐2025 MHz and 2110‐2220 MHz including satellite components on a worldwide basis for the use of Future Public Land Mobile Telecommunication Systems (FPLMTS).   Since it was scheduled to start service around the year 2000, FPLMTS was later changed to IMT‐ 2000 which stands for International Mobile Telecommunication‐2000. At time of writing, it has been agreed that spectrum currently allocated for exclusive use by IMT‐2000 technologies will now be known as just “IMT spectrum”.

IMT‐2000 is also known as the third generation mobile systems. In order to differentiate IMT‐ 2000 from the former generation, Table 1 below compares the significant differences between the 2nd and 3rd generation mobile systems.

Table 1- Differences between the 2nd and 3rd generation mobile systems

ITU's Radio communication Sector (ITU-R) has completed the assessment of six candidate submissions for the global 4G mobile wireless broadband technology, otherwise known as International Mobile Telecommunications – Advanced (IMT-Advanced) in October 2010. Harmonization among these proposals has resulted in two technologies, "LTE-Advanced" and "WirelessMAN-Advanced" being accorded the official designation of IMT-Advanced, qualifying them as true 4G technologies. ITU-R Working Party 5D, which is charged with defining the IMT-Advanced global 4G technologies, decided on these technologies for the first release of IMT-Advanced. These technologies will now move into the final stage of the IMT-Advanced process, which provides for the development in early 2012 of an ITU-R Recommendation specifying the in-depth technical standards for these radio technologies.

Pre 4G technologies such as mobile WiMax and the first release Long Term Evolution (LTE) has been in market since 2006 and 2009 respectively and often branded as 4G in marketing material. The current versions of these technologies do not fulfil the IMT- Advanced requirement. IMT- Advanced compliant version of the above two standards are under development and called “LTE Advanced” and “WirelessMAN Advanced” respectively. ITU has decided that “LTE Advanced” technology based on 3GPP release 10 and “WirelessMAN Advanced” technology based on IEEE 802.16m should be accorded the official designation of IMT- Advanced. Both LTE-Advanced technology and the IEEE WirelessMAN -Advanced technology make use of same key technologies viz. Orthogonal Frequency Division Multiple Access (OFDMA), Multiple Input Multiple Output (MIMO) and System Architecture Evolution (SAE).

IMT‐Advanced systems are mobile systems that include the new capabilities of IMT that go beyond those of IMT‐2000. The following Table 2 compares key features between IMT‐2000 and IMT‐Advanced.

Table 2- compares key features between IMT‐2000 and IMT‐Advanced.

#### 2G network architecture

The 2G network architecture is defined in the GSM specifications and it can be grouped into four main areas:

- Network and Switching Subsystem (NSS)

- Base-Station Subsystem (BSS)

- Mobile station (MS)

- Operation and Support Subsystem (OSS)

The different elements of the GSM network operate together and the user is not aware of the different entities within the system.

As the GSM network is defined but the specifications and standards, it enables the system to operate reliably together regardless of the supplier of the different elements.

A basic diagram of the overall system architecture for the 2G GSM mobile communications system includes four major elements which are shown below:

Figure.2- 2G GSM architecture

#### 3G network architecture

3G can in many aspects be looked upon as an extension to GSM and GPRS. The greatest changes are related to the access part of the network. The access network, called UMTS. Universal Terrestrial Radio Access Network (UTRAN), consists of base stations and base stations controllers.

The base stations are called Node B. A Node B can support FDD mode, TDD mode or dual mode operation. Several base stations are managed by a Radio Network Controller (RNC). The RNC is responsible for the Handover decisions that require signaling to the UE. A logical view of the network is shown in Figure 2.

Figure.3- 3G network architecture

#### 4G network architecture

The high-level network architecture of LTE is comprised of following three main components:

- The User Equipment (UE).

- The Evolved UMTS Terrestrial Radio Access Network (E-UTRAN).

- The Evolved Packet Core (EPC).

The evolved packet core communicates with packet data networks in the outside world such as the internet, private corporate networks or the IP multimedia subsystem. The interfaces between the different parts of the system are denoted Uu, S1 and SGi as shown below:

Figure.4- 4G interface architecture

The architecture of evolved UMTS Terrestrial Radio Access Network (E-UTRAN) has been illustrated below:

Figure.5- 4G network architecture

The E-UTRAN handles the radio communications between the mobile and the evolved packet core and just has one component, the evolved base stations, called eNodeB or eNB. Each eNB is a base station that controls the mobiles in one or more cells. The base station that is communicating with a mobile is known as its serving eNB.

LTE Mobile communicates with just one base station and one cell at a time and there are following two main functions supported by eNB:

- The eBN sends and receives radio transmissions to all the mobiles using the analogue and digital signal processing functions of the LTE air interface.

- The eNB controls the low-level operation of all its mobiles, by sending them signaling messages such as handover commands.

Each eBN connects with the EPC by means of the S1 interface and it can also be connected to nearby base stations by the X2 interface, which is mainly used for signaling and packet forwarding during handover.

A home eNB (HeNB) is a base station that has been purchased by a user to provide femtocell coverage within the home. A home eNB belongs to a closed subscriber group (CSG) and can only be accessed by mobiles with a USIM that also belongs to the closed subscriber group.

# Chapter 3: Spectrum utilization

## Introduction

It is widely recognized that the radio spectrum is defined along three dimensions: space, frequency and time.  The term "spectrum utilization" refers to the amount of information (usually measured in bits) that is being carried by a spectrum unit (measured in m2. Hz. Sec.). The maximum achievable depends on many factors ranging from the physical propagation conditions to the state of technology and system design.

## Spectrum utilization theory

Spectrum utilization factor, U, is defined to be the product of the frequency bandwidth, the geometric (geographic) space, and the time denied to other potential users:

Where:

B:	frequency bandwidth

S:	geometric space (usually area)

T:	time.

The geometric space of interest may also be a volume, a line (e.g. the geostationary orbit), or an angular sector around a point. The amount of space denied depends on the spectral power density. For many applications, the dimension of time can be ignored, because the service operates continuously. But in some services, for example, broadcast and single channel mobile, the time factor is important to sharing and all three factors should be considered simultaneously, and optimized.

The measure of spectrum may be computed by multiplication of a bandwidth bounding the emission (e.g. occupied bandwidth) and its interference area, or may take into account the actual shape of the power spectrum density of the emission and the antenna radiation characteristics.

Traditionally, radio transmitters have been considered the users of the spectrum resource. They use the spectrum-space by filling some portion of it with radio power – so much power that receivers of other systems cannot operate in certain locations, times and frequencies because of unacceptable interference. Notice that the transmitter denies the space to receivers only. The mere fact that the space contains power in no way prevents another transmitter from emitting power into the same location; that is, the transmitter does not deny operation of another transmitter.

Receivers use spectrum-space because they deny it to transmitters. The mere physical operation of the receiver interferes with no one (except as it inadvertently acts as a transmitter or power source). Even then the space used physically is relatively small. However, the authorities deny licenses to transmitters in an attempt to guarantee interference-free reception. The protection may be in space (separation distance, coordination distance), in frequency (guard bands) or even in time (in the United States of America, some MF broadcasting stations are limited to daylight operation). This denial constitutes “use” of the space by the receiver. The radio astronomy bands are a familiar example of the recognition of receiver use of the spectrum space.

One way to incorporate these facts into a unit of measure of spectrum space is to partition the resource into two spaces – the transmitter space and receiver space – and define dual units to measure the usage of each space. Where simplicity is most important, the two units can be recombined into a single measure for system use.

Further information concerning the general approach to calculate the spectrum utilization factor may be found in Chapter 8 of the National Spectrum Management Handbook.

## Spectrum utilization measurement

### Spectrum measurement

Spectrum measurements can be divided into the five phases shown in figure 6. It is important to design and implement each phase properly in order to obtain meaningful data and to communicate this data to the interested users. Each phase and its unique challenges are discussed in the following sections.

Figure.6- Spectrum measurements phases

#### Filtering and sampling

The creation of an interference map is a multidimensional problem covering the spectral, temporal and spatial domains. The most accurate interference map for a certain frequency would be the one where energy measurements are continuously taken at all possible locations. However, there are practical limitations due to sampling in each dimension. A sample is defined as an observation at a specific time, frequency, and location. Each sample is quantized. Sampling is usually limited in order to maintain the amount of data at a manageable level considering the requirement to distribute and combine the information in a reasonable amount of time. In general, sampling requirements are dependent on the objective of the measurements. The Nyquist sampling theorem defines the time domain requirements if there is a need to reconstruct a signal, i.e., the sampling frequency must be at least twice as high as the bandwidth of the input signal twice as high as the bandwidth of the input signal

#### Interpolation

The creation of an interference map requires the use of interpolation techniques to estimate values at points that have not been measured. The interpolation methods can be divided into global methods that use all the available data for estimation and local methods that use only the information in the vicinity of the measured points. In addition, the methods are either exact, meaning that the predicted values at the measurement points are the known values, or inexact, where the observed values and the interpolated values for a certain point are not necessarily the same. There are numerous spatial interpolation methods that can be used to create continuous spatial data over the region of interest; 72 methods or sub-methods are mentioned in. There is no single preferred method since the selection of a method needs to be based on the actual data, the level of accuracy required, and the time and/or computer resources available. We will concentrate on the methods that have been proposed specifically for interference map creation. The classification of information will be developed based on.

#### Reduction of data using metric

Measurements often provide huge amounts of data, especially if they are conducted over a long term. For example, the various spectrum occupancy measurements at the Illinois Institute of Technology Spectrum Observatory with less than ten detectors occupy many terabytes of data storage. There is therefore often a need to reduce the amount of data to obtain meaningful information from the measurements within a reasonable timeframe. In general, the amount of measurement data over an area of interest grows uncontrollably large due to frequent sampling in all domains.

#### Data storage and management

Large-scale data storage and management systems have developed rapidly during the last decade due to both the rapid decline in the cost of the storage systems and the ever increasing demand for data based on the vast number of sensory systems, devices and services producing huge chunks and streams of information. This data in turn has allowed researchers and businesses to analyze various aspects of our natural and human environment and to build models for inference in different areas in society Analysis and management of the spectrum with related spatial, temporal, and contextual aspects of data requires advanced, yet specific hardware and software solutions to be able to store, access and process data, possibly in a real-time manner. Platforms based on cloud computing and high performance computing that are using, e.g., graphical processing units (GPU) in conjunction with distributed database storage, processing and access are becoming mainstream in the development of anytime, anywhere accessible services.

#### Visualization

In addition to well-chosen metrics and related numbers, one needs to visualize the information properly. The public assumes the existence of meaningful spectrum visualizations to enable the prediction of what communication services are available, where and at what times. This is also important information for operators who try to optimize their network and serve their customers as well as possible. Visualization also provides the means to support new spectrum allocations and improved spectrum sharing. It provides answers to the critical question: How to efficiently communicate the stored information to users, regulators and network engineers.

Engineers and scientists are often trained to read and understand spectrum images. However, there is a diverse audience, including policymakers and the public, with no professional training on how to read spectrum images or think about spectrum [108]. Therefore, it is often very important to use metaphors to produce images that are interpretable by all. Colors and well labeled charts are typically used to create graphical metaphors of invisible spectrum situations, see e.g., Figure. 7.

Figure.7- Sample of spectrum image

### Evaluate of the Spectrum Utilization

When Spectrum utilization is joined with the time factor, it is called spectrum occupancy of a radio channel. Spectrum occupancy provides information on the actual level of usage of individual frequency channels.

This is defined as the percentage of the time that a channel is occupied by such messages, over a given period.

In section number 3-2, the theory of utilization is shown, however in practical cases, measurement of utilization has been defined by Duty Cycle parameter per percent, which specifies the fraction of time the band is used in the area that the transmitter causes interference in the specific space.

The duty cycle has been calculated as ratio of number of samples with power level superior to power threshold Np>threshold and the total number of samples NTotal as indicated in Eq.1

Specific duration

&

Area

Duty cycle = NP>threshold / NTotal				

Figure.8- position of monitoring station

The measurement method is based on the energy detection principle. The RF signal has been captured by a wideband antennas and continuously scanned by a spectrum analyzer. The system should measure all of directional in a specific duration. (For example a six-day).

Determination of the decision power threshold is a critical issue. An excessive threshold may lead to overlooking potential activity within the band, but on the other hand, too low values may result in false alarm due to the background noise.

In order to overcome this problem, different thresholds for different frequency bands have been defined, depending on the level of the background noise. The power threshold that determines the occupancy of a frequency sample has been set in most cases to 7 dB above the average value of the background noise measured within the band of interest. 7 dB has been chosen as the optimal value based on long term observation of the background noise.

For instance, analytical results for individual frequency bands, figure 9 is shown result of measuring in frequency bands in GSM 800 MHz

Figure.9- result of measuring in frequency bands in GSM 800 MHz

# Chapter 4:  Spectrum Utilization Efficiency

## Introduction

As the spectrum demand increases, efficient spectrum resource use in a timely manner becomes necessary. Although many countries have researched and analyzed the current status of spectrum utilization periodically, it was challenging to produce an appropriate policy that considers future demand and supply comprehensively, because of the limitations of fragmentary and qualitative analyses.

As a result, some countries have recently applied a method of discovering candidate bands and evaluating their priority in order to efficiencies of using of spectrum systematically in the long-term planning process.

## Theory of Spectrum utilization efficiency (SUE)

According to the definition of SUE (or spectrum efficiency as a shortened term) of a radiocommunication system, it can be expressed by a complex criterion:

SUE  {M, U}    {M, BST}		(2)

Where:

M:	useful effect obtained with the aid of the communication system in question

U:	spectrum utilization factor for that system defined in 3-2.

If necessary, the complex spectrum efficiency indicator may be reduced to a simple indicator: the ratio of useful effect to spectrum utilization factor:

(2а)

## Evaluate of SUE in SATRC’s countries based on received answers

For giving information, a questionnaire was prepared and for circulating, it was sent to all of SATRC members. This questioner has 2 main parts, first about evaluating of SUE and next is spectrum audit. In the following, first section in 5 parts will be analyzed.

- policies/regulatory measures about spectrum utilization

| Country | Answer |
| --- | --- |
| Pakistan | Initially there were De-Regulation Policy for the Telecommunication Sector 2003, and Mobile Cellular Policy 2004. These two government approved policies provided guiding principles and basic frameworks to FAB for efficient spectrum utilization and management. Afterwards, through promulgation of Telecom Policy 2015. Spectrum management principles were suitably addressed therein. Based on the said principles enumerated in Telecom Policy 2015P Pakistan Rolling Spectrum Strategy was published in 2020. |
| India | The National Frequency Allocation Plan is used for planning spectrum utilization in the country. |
| Bhutan | The Information Communication and Media Act 2018 which also highlights the Usage of radio frequency spectrum. We also have the National Radio Rules and Regulations 2021 And the National Frequency Allocation Plan 2021. |
| Sri Lanka, Bangladesh, Nepal, Afghanistan, and Maldives | Not specific policies/ regulatory |

- Carry out of the evaluation of spectrum utilization

| Country | Answer |
| --- | --- |
| Pakistan | The initiatives to carry out evaluation of spectrum utilization varies on case to case basis depending upon the incumbent users, services being offered, technology, areas of operation and potential alternate future use of the band under consideration. |
| Sri Lanka | In IMT services, TRC introduces coverage obligations which need to be fulfilled within certain time period. |
| India | Whenever new bands are identiﬁed in the World Radiocommunication Conferences for any radiocommunication services, Government carries out necessary review of the occupancy of bands for making available spectrum for such services. |
| Bangladesh, Nepal, Bhutan, Afghanistan, and Maldives | no regulatory initiative to carry out |

- How utilization of spectrum evaluated

| Country | Answer |
| --- | --- |
| Pakistan | The utilization of spectrum per radio service(s) is evaluated based on detailed spectrum monitoring surveys, through utilization of spectrum monitoring system. That also involves appropriate access to the operational system of incumbent users or licensees. The evaluation of utilization of spectrum is based on taking into account the relevant ITU-R recommendations such as ITU-R SM. 1046-3. |
| Sri Lanka | TRC evaluates the achievement of coverage obligations in regular intervals. The operator shall give a surety in terms of bank guarantee for the obligation. If the operator fails to fulfill the obligation, TRC forfeits the bank guarantee as a penalty. If the operator is continuing the failure to meet the obligation the frequency shall be withdrawn. |
| India | Concurrent audit was carried out recently in India in which the spectrum utilization of the major users was audited and the recommendations provided are being evaluated and implemented. The preform for audit is attached herewith. |
| Bhutan | We have not carried out the spectrum utilization based on per radio service(s). However, we do check the spectrum occupancy once the spectrum has been assigned to the licensees. |
| Afghanistan | Currently there are more than 35000 frequency permit letters assigned to different organization for utilizing the spectrum for different services, based on their request and normally renewed on annual basis. Majority of these permit letters belong to MNOs, ISPs, TTSPs. It is very important to be mentioned that the information in this paragraph is very general, the spectrum utilization evaluation is not yet in practice. |
| Bangladesh, Nepal, and Maldives | no regulatory initiative to carry out |

- Border (high and low limit) in the evaluation of spectrum utilization

| Country | Answer |
| --- | --- |
| Pakistan | The border in the evaluation of spectrum utilization varies on case to case basis depending upon the incumbent users, services being offered, technology, areas of operation and potential future use of the band under consideration. Similarly, the end objective behind the spectrum utilization evaluation also has a bearing on the border being decided. |
| India | Same as a and b sections |
| Bhutan | We have not carried out the spectrum utilization based on per radio service(s). However, we do check the spectrum occupancy once the spectrum has been assigned to the licensees. |
| Sri Lanka, Bangladesh, Nepal, Afghanistan, and Maldives | no regulatory initiative to carry out |

- Affect result of the evaluation in spectrum managing decisions

| Country | Answer |
| --- | --- |
| Pakistan | Yes, the results of the spectrum evaluation affects spectrum manager decisions. In the presence of detailed spectrum evaluation report, the spectrum manager can make decision supported through solid evidence and justification, |
| Sri Lanka | Results of the evaluation will help to formulate future policies and regulatory frameworks. |
| India | Necessary steps for surrender/vacation by the incumbent users in accordance with Audit |
| Bangladesh, Nepal, Bhutan, Afghanistan, and Maldives | no regulatory initiative to carry out |

## Evaluate of SUE in IMT networks based on Iran’s experience

The proposed method for calculating SUE, which has a completely practical approach based on the parameters and KPIs of mobile networks, and it has been tried to become a suitable candidate for completing the existing theoretical methods in the ITU’s recommendation.

In the rest of this chapter, the proposed method is presented in detail in various types of cellular communication networks in different generations.

### 2G Network

According to the definition of SUE, in 2G networks, it is calculated based on two types of services, including CS and PS in GSM, GPRS and EDGE technologies.

In this way, for CS services in GSM, the SUE index is equal to Erlang/MHz/Sq.a

For PS services in 2.5G (GPRS) and 2.75G (EDGE) technology, instead of Erlang, data volume and payload in terms of Mbit are used, for calculating and evaluating SUE, it should be taken into account following steps:

- Define a cluster. It can be selected as a cell, partial of city or full  of city’s geo,

- Calculating of area of the cluster as m2,

- Define cells in the cluster,

- Define number of TRXs  in each of the cells and its common/single BCSH,

- In the busy hour of cluster, evaluate payload and Erlang with consider half/full rate that consume by users in the cluster.

- Calculating amount of spectrum that assign to TRXs in the cluster. Also with consider reuse grade and TRX‘s bandwidth in this technology that is 200Khz, we can calculate amount of spectrum as theoretically.

After define all of above parameters, we can calculate SUE base on the formula that are following:

SUE=Total Erlang or payload carried in the cluster (Number of channels per TRX×Number of cells×200Khz)×Area of the cluster

### 3G Network

Calculating 3G networks is the same as 2G networks, calculated based on two types of services, CS and PS. So, for calculating and evaluating SUE, it should be taken into account following steps:

- Define a cluster.

- Calculating of area of the cluster as m2,

- Define number of cells in the cluster,

- In the busy hour of cluster, evaluate payload and Erlang that transferring in the cluster

- Calculating amount of spectrum.

Now, we can calculate SUE for 3G network with following formula:

SUE=Total Erlang or payload carried in the cluster (Number of Cells×5Mhz)×Area of the cluster

### 4G Network

For calculating 4G networks, we should consider just PS and should be taken into account 3G steps. For amount of spectrum that used in the cluster in this technology, we should consider this note that, cells can be assigned bandwidth 5, 10, 15, and 20 MHz

Now, we can calculate SUE for 4G network with following formula:

SUE=Total Erlang or payload carried in the cluster (Number of Cells×BW 5, 10, 15 and 20MHz)×Area of the cluster

# Chapter 5: Evaluate of relative spectrum utilization efficiency (RSE)

## Theory of RSE

The concept of RSE can be used effectively to compare the spectrum efficiencies of two similar types of radio systems providing the same service.

RSE is defined as the ratio of two spectrum efficiencies, one of which may be the efficiency of a system used as a standard of comparison. Hence,

RSESUEa / SUEstd	(3)

Where:

RSE:	relative spectrum efficiency ratio of SUEs)

SUEstd:	SUE of a “standard” system

SUEa:	SUE of an actual system.

The likely candidates for a standard system are:

–	The most theoretically efficient system,

–	A system which can be easily defined and understood,

–	A system which is widely used.

The RSE will be a positive number with values ranging between zero and infinity. If the standard system is chosen to be the most theoretically efficient system, the RSE will typically range between zero and one.

As an example, the most theoretically efficient system may be characterized according to the principles of information theory.

## Evaluate of standard SUE in SATRC’s countries based on received answers.

For giving the information, the questionnaire has a question about this section, so in the following, result of the survey is mentioned:

- mechanism/method to get relative information for the evaluation

| Country | Answer |
| --- | --- |
| Pakistan | The method/ mechanism evaluation of spectrum utilization varies on case to case basis depending upon tine incumbent users, services being offered, technology, areas of operation and potential future use of the band under consideration. Similarly, the end objective behind the spectrum utilization evaluation exercise also affects the method/ mechanism being adoptee. |
| Sri Lanka | TRC requests operators to submit related information and TRC verifies those information in random basis. |
| Bhutan | From ITU recommendations and suggestions made by the APT/ STARC working group. |
| India | Same as b and c in the previous section |
| Bangladesh, Nepal, Afghanistan, and Maldives | no regulatory initiative to carry out |

## Evaluate of standard SUE in IMT networks based on Iran’s experience

In the continuation of this proposal, the calculation of RSE, which is an important issue in various communication technologies, is planned. Using this index, we can evaluate the efficiency of the frequency bands used by operators.

### 2G Network

Using the received information from the operators in the previous section, perform the following steps and calculate the amount of RSE in this technology:

- CS Service:

- Define number of TRX with consider single/common BCCH and SDCCH that received from operator,

- With consider number of TRX, calculate number of TCH time slot in the cluster,

- Search in ErlangB table with consider number of (ii) and percent of grade of service.

- PS Service

For calculating PS in this section, first we should consider spectral efficiency coefficient (b/Hz/Cell) in this technology, in the proposal this amount is 0.09 b/Hz/TRX. And following bellow steps:

Spectral_Efficiency = 0.09 			(b/s/Hz/TRX)

BW_Hz = 200*1000 		 		Hz

Throughput = Spectral_Efficiency*BW_Hz 	 (b/s/Cell)

Payload_b = Throughput*3600 			 (bit/Cell)

Payload_MB = Payload_b/1000000/8 	 (MB/Cell)

SUEstandard=Number of TRX*Payload_MB (Number of channels per TRX×Number of cells×200KHz)×Area of the cluster

### 3G Network

In this section is same as 2G network, with received information from operator and following bellow steps, we can calculate standard SUE:

- CS Service:

- Define number of cells,

- According to ErlangB table, considered, 8.76 ErL. Per cell, is considered for this technology.

SUEstandard=number of cells*8.76(Number of Cells×5MHz)×Area of the cluster

- PS Service

For calculating PS in this section, first we should consider spectral efficiency coefficient (b/Hz/Cell) in this technology, in the proposal this amount is 0.5 b/Hz/Cell. And following bellow steps:

Spectral_Efficiency = 0.5 			(b/s/Hz/Cell)

BW_Hz = 5*1000000 		 		Hz

Throughput = Spectral_Efficiency*BW_Hz 	 (b/s/Cell)

Payload_b = Throughput*3600 			 (bit/Cell)

Payload_MB = Payload_b/1000000/8 	 (MB/Cell)

SUEstandard=Number of cells*Payload_MB (Number of cells×5MHz)×Area of the cluster

### 4G Network

For calculating PS in this section, following bellow steps:

Spectral_Efficiency = 1.4 			(b/s/Hz/Cell)

BW_Hz = BW*1000000 		 	BW: 5, 10, 15, 20 MHz

Throughput = Spectral_Efficiency*BW_Hz 	 (b/s/Cell)

Payload_b = Throughput*3600 			 (bit/Cell)

Payload_MB = Payload_b/1000000/8 	 	(MB/Cell)

SUEstandard=Number of cells*Payload_MB (Number of cells×BW)×Area of the cluster

# Chapter 6: Spectrum audit

## Background

Currently, the use of radio equipment is expanding rapidly, therefore, inspection and control of the use of radio spectrum is one of the important responsibilities of spectrum management, whose main goal is to promote the effective use of radio spectrum. In the inspection, by visiting the radio stations, it is ensured that the conditions of the license granted to the equipment, the general conditions of the establishment of the station and the way of operating the spectrum are met, and if necessary, the law is applied. Also, spectrum management needs to ensure the elimination of interference, so inspection also helps to prevent interference.

Inspection is especially important when a radio station has been identified as an interference factor. On the other hand, radio equipment must be compatible with license conditions such as frequency allocation, transmitter power, antenna specifications, antenna height, frequency deviation and occupied bandwidth.

Unauthorized use of radio equipment may lead to interference on the signal of legal users of the spectrum, which, for example, in emergency services, this interference endangers life and property and the safety of people. Since the spectrum management system is responsible for preventing harmful interference, it must establish an inspection system to check intentional or unintentional violations and ensure that the radio equipment is installed correctly and that its operation does not lead to harmful interference.

The objective of the audit was to assess the adequacy and operating effectiveness of the management control framework in place for spectrum compliance.

The audit scope focused on the spectrum compliance management control framework, including:

- Compliance activities designed and carried out in national headquarters and in the regional offices.

- Governance and oversight;

- Planning and reporting;

- Processes, procedures and tools;

- Training; and

- Continuous improvement.

## Audit Objectives

The objectives of the audit are:

- Using of spectrum that assigned by administration to user (Government Ministries/ Departments/ Agencies) is accordance with relative license’s parameters,

- Management and monitoring of activities in spectrum for:

  - Identify unused spectrum by authorized users,

  - Measurement utilization of spectrum by authorized users,

  - Identify invalid activities in spectrum.

- Technical management of the spectrum resource is necessary to ensure that as many users and as many uses can be accommodated as possible without harmful interference to one another,

- Complete licensing audit would greatly assist in ensuring that a proper economic return is provided to the Government for the use of the radiofrequency spectrum.

- To improve spectrum availability, begin with knowing how well the spectrum is being used (with respect to quantum, geography and time) and by whom.

- Examine whether utilization of spectrum assigned administratively to user Government Ministries/ Departments/ Agencies was optimal.

- To know the surplus spectrum available with the government user agencies.

- Whether more quantity of spectrum is being used because of deployment of old technology/ equipment.

- Whether use of new/digital technology will require lesser quantity of spectrum and render a substantial part of the spectrum free.

- What may be the cost of replacing old technology/ equipment viz a viz economic gain accrued through freeing up of the spectrum.

- Opportunity cost of the Spectrum. (Economic value obtained if the same spectrum is used for other purpose).

- Audit should also be conducted about proper and timely assignment of spectrum by the Spectrum Managers in a transparent manner, particularly that spectrum which are assigned administratively.

- How much spectrum has not been assigned to the user agencies (public or private) and is idling with the Spectrum Manager, generating no economic value.

- To examine whether database of frequencies assigned to Government Users is properly maintained.

- Examine measures taken by the spectrum Managers for adequate management and monitoring of spectrum utilization.

- Examine efforts made by user departments and Spectrum Managers to enhance spectral efficiency and release/ surrender unused spectrum.

- Spectrum Audit should be conducted by an independent agency through a transparent and objective oriented process.

## Audit Methodology

The methodology included various procedures to address the engagement's objective. This included, but was not limited to, review of documentation, interviews, sample files review, site visits to regional offices, process and system walkthroughs.

Sample of compliance activity files, equipment, and fleet selected for review, included:

- Compliance activity files to assess whether processes and practices are comparable across the regions.

- Competence of inspectors check.

- Equipment match (such as Radio-frequency Receivers, Spectrum Explorers and Analyzers, vehicles).

The audit should be conducted on the basis of records/ information made available by the respective wings/ divisions/ zones/ authorities of the Ministries/ Departments/ Agencies.

Inspectors need to have access to public/special documents for the audit. If there are no obstacles, the audit results can be published on the regulatory website.

## Sources of Audit Criteria

The major sources of audit criteria used in the Audit include the following:

- National policies, decisions, recommendations and regulations.

- ITU Radio Regulations and relevant ITU recommendations.

- International practices of Spectrum Management

- Guidelines/ procedures for Spectrum Management

## Audit methods

### ITU

ITU recommends five methods for audit. First, audit all of radio stations that usually, with the priority of audit of newly licensed stations and audit of stations, at least once during their license period. The second method is triggered audit, which is carried out in cases of complaints of interference, or detection of violations by spectrum monitoring. Sample method, stations are inspected, this method is similar to the random inspection method. Limited way, the method checks only one specific case of the inspection parameters of interest to the management, such as the output power of the transmitter. Some licenses have the title of high-risk and are related to radio stations with a high potential to cause interference, and the inspection of these stations is called the risk-based inspection method.

### CEPT

Inspection methods in CEPT member countries are carried out with four methods of interference, random, case by case and rounds. All of methods are same as ITU. The focus of audit at CEPT is mostly on private mobile radio networks and interference reports. The purpose of random audit is to ensure the operation and installation of the equipment in accordance with the license conditions and, as a result, to prevent interference by the equipment.

### FCC

The inspection process at the FCC is conducted randomly or when interference is reported. Each station is obliged to carry out all the necessary measurements for the inspection and provide that information to the inspector whenever the inspector comes to visit the station. In each inspection, the licensee must allow the inspection of the station during every hour of its operation, and if certain checks and measurements are required by the inspector, the licensee must provide suitable conditions for the inspector.

### SATRC countries

Collected information in this section by the questioner, analysis and its result mentioned in the following:

- Doing spectrum Audit

| Country | Answer |
| --- | --- |
| Pakistan | MoIT, on recommendations of FAB and PTA, will prepare and publish a rolling spectrum strategy to be published every year that provides a program for the succeeding three years from the date of publication. The Spectrum Strategy envisages to identify for the succeeding period, a plan for existing spectrum allocation audit. The spectrum audit is carried out on case to case basis necessitated through process of spectrum rearming or repurposing exercise |
| Sri Lanka | Just started with broadcasting service |
| India | Concurrent audit was carried out recently in India in which the spectrum utilization of the major users was audited and the recommendations provided are being evaluated and implemented. The preform for audit is attached herewith. |
| Bhutan | Not as of now. We only do spectrum occupancy monitoring. |
| Bangladesh, Nepal, Afghanistan, Maldives | No |

- Details of audit

| Country | Answer |
| --- | --- |
| Pakistan | - Spectrum audit can be conducted for any band and or service. - The spectrum audit is mainly conducted for spectrum occupancy, utilization and associated other technical parameters - The spectrum audit is conducted by FAB. However, the consultation also involves MoIT&T, PTA and the incumbent users |
| Sri Lanka | - TRC started with broadcasting service and we are formulating new rules to do spectrum audit in other services also. - In broadcasting TRC measures transmitter power, deviation, bandwidth, coverage etc. - By the regulator do it. |
| India |  |
| Bangladesh, Bhutan, Nepal, Afghanistan, Maldives | No |

### Iran’s experience

In Iran, 3 methods have been carried out by CRA:

- The first method is the inspection method based on the notification of interference. In this method, after the notification of interference and during the stages of interference analysis and identification of the interferer, it is necessary to send inspectors to the location of the interferer and inspect the station and equipment and apply the law.

- The second method is based on the spectrum monitoring results. After measuring, observing the violation and non-compliance of the parameters with the license conditions, the monitoring station sends a report to the management and the inspectors are sent to the location of the offending station to perform more accurate measurements.

- According to the ITU recommendation, all scientific research stations should be inspected annually, and audit are required for radio station before granting or renewing an operating license. Inspection every three months of at least 4 companies that produce, import or distribute telecommunication products is also recommended. In addition to this inspection of other stations, a combination of random and rounds is suggested. This method is very useful when the number of stations is large and inspection of all stations is not practical due to budget, staff and other factors.

# Chapter 7: Conclusion

## Aim of evaluating spectrum utilization

Economy is one of the important constituents of efficient spectrum utilization. Apart from the technical characteristics of radio stations, economy of spectrum utilization is determined primarily by the degree to which the method used by the planning (or coordinating) authority to assign frequencies to operational systems corresponds to the optimum (or near-optimum) method.

Evaluating of spectrum utilization as a spectrum management tools plays a major role in increasing the country’s social and economic well-being by maximizing the use of the spectrum for wireless applications. It is strongly emphasized that economic benefit in this sense must be used in a broad context rather than that of merely increasing license revenue. Implementation of the result of utilization in spectrum management, can be achieved maximize the social and economic benefits during the licensing process that:

  - ensures use of the best available frequency range appropriate to the application with the highest efficiency permitted by technology;

  - promotes sustainable competition among service providers;

  - leads to higher density usage within service allocations by using spectrum efficient technologies, frequency re-use, improved frequency planning models, improved sharing criteria and traffic density projections;

  - promotes new radio service entrants;

  - Identifies, quantifies (to the extent possible) and maximizes the social benefits accruing to the licensing strategy under consideration.

## Audit

The Spectrum program is making progress in implementing its National Compliance Framework. Future efforts will be required in order to ensure a nationally consistent approach to all compliance activities, including enhancing the methodology for planning compliance activities, further defining procedures, building quality controls over the capture of information, and producing more comprehensive annual reporting.

### Audit Observations

The following observations are generally made while conducting the audit:

  - Spectrum allotted/assigned to Government Departments in various bands, including IMT bands, is either sub-optimally utilized or not utilized at all for years.

  - Utilization of spectrum for Analogue Terrestrial Television (ATT) and Digital Terrestrial Television (DTT) remains sub-optimal.

  - Spectrum in Mid-Band available with the Government agencies can easily be vacated and re-farmed for commercial telecom services optimal utilization.

  - The National Frequency Register (NFR) is generally not updated with the cancellations and surrender of frequency authorization, change of frequency, etc. and there is mismatch in the database, regarding spectrum assigned to Government Departments/ PSUs/ Agencies.

  - Ineffective Wireless Monitoring due to lack of modern monitoring equipment.

  - Lack of any institutional mechanism or any apparent MIS for monitoring of spectrum use by the Government Departments/Agencies.

### Key Audit Recommendations

The following steps may be taken to identify the unused spectrum and improve the spectrum utilization:

  - Spectrum Managers may coordinate with Government users to provide spectrum in substitute bands, conduct spectrum harmonization exercise and take back the spectrum that have commercial scope of utilization.

  - Spectrum Managers, in consultation with respective Ministry, may take action to examine the spectrum utility in Digital Terrestrial TV bands.

  - Spectrum Managers should devise a policy for allotment and assignment of spectrum for Captive Users/other commercial usage, in alignment with stated policy and procedures in a transparent manner.

  - Spectrum Managers should review the pricing mechanism of Spectrum for Captive Users in order to incentivize Ministries/ Departments/ Agencies to maintain spectrum management discipline and nudge them to surrender unutilized spectrum.

  - Spectrum Managers should have a permanent set up comprising all stakeholders for periodical review of spectrum planning, spectrum availability, allotments/assignments, pricing, etc. to expedite decisions for efficient management and optimal utilization of spectrum.

  - Spectrum Managers should also ensure that a MIS is put in place to get details of actual spectrum utilized by Government user Departments and Agencies.

  - Spectrum Managers need to acquire and implement an industry standard Automated Frequency Management System (AFMS). Further, Spectrum Managers may update the existing database on spectrum users on priority before migration to the new system and make available the updated database on a timely basis to their Monitoring Organization for efficient monitoring of assigned Spectrum.

  - Spectrum Managers should urgently upgrade the infrastructure and equipment of its Monitoring Organization to strengthen the monitoring of spectrum.

### Benefits of Spectrum Audit

The following will be the accrued benefits of the spectrum audit:

  - The Spectrum Audit will give insight into the different aspects of spectrum management that needed attention.

  - It will help in identifying the frequency chunks that are not being utilized.  Will help in re-farming the spectrum in key frequency bands in demand.

  - It will find spectrum to introduce new services such as next generation IMT / Wi-Fi in desired spectrum bands.

  - It will strengthen the Frequency planning and assignments, including providing information on spectrum occupancy, usage patterns etc.

  - It will justify and highlight the importance of Wireless Monitoring for efficient spectrum management.

  - The spectrum audit will re-emphasize the need of a robust database of spectrum users.

# Chapter 8: Questions and Answers

## Questions

Part: 1

Question 1:

Do you have any policies/regulatory measures about spectrum utilization in your country? (Please mention summary information or attach document)

Question 2:

Is there any regulatory initiatives to carry out the evaluation of spectrum utilization in your country? If yes, which radio service(s):

Question 3:

How is the utilization of spectrum per radio service (s) mentioned in answer Q.1 evaluated? (Please write a summary of activities and your experience).

Question 4:

Do you have any mechanism/method to get relative information for the evaluation?

Question 5:

Do you have any border (high and low limit) in the evaluation of spectrum utilization? (Please mention detail)

Question 6:

Does the result of the evaluation affect spectrum manager decisions in your country? (Please mention information and procedure)

Part: 2

Question 1:

Is spectrum Audit conducted in your country?

Question 2:

If your answer to Question number 1 is yes, then

- Whether audit of spectrum, held by telecom / broadcasting service providers (terrestrial /satellite) and all other entities, including public utilities, such as emergency services, national defense, railways is conducted in your country?

- On what parameters, is the spectrum audit conducted?

- The work of Spectrum audit is done by the regulator/ licensor or is assigned to some other agency. Please provide relevant documents and details

Question 3:

If your answer to Question number 1 is 'No', what are the methods used to assess the effective spectrum utilization in your country?

## Answers

### Pakistan

Part 1:

Answer 1:

In Pakistan, initially there were De-Regulation Policy for the Telecommunication Sector 2003, and Mobile Cellular Policy 2004. These two government approved policies provided guiding principles and basic frameworks to FAB for efficient spectrum utilization and management. Afterwards, through promulgation of Telecom Policy 2015. Spectrum management principles were suitably addressed therein. Based on the said principles enumerated in Telecom Policy 2015P Pakistan Rolling Spectrum Strategy was published in 2020.

Answer 2:

The initiatives to carry out evaluation of spectrum utilization varies on case to case basis depending upon the incumbent users, services being offered, technology, areas of operation and potential alternate future use of the band under consideration.

Answer 3:

The utilization of spectrum per radio service(s) is evaluated based on detailed spectrum monitoring surveys, through utilization of spectrum monitoring system. That also involves appropriate access to the operational system of incumbent users or licensees. The evaluation of utilization of spectrum is based on taking into account the relevant 1TU-R recommendations such as ITU-R SM. 1046-3.

Answer 4:

As explained above in reply to Question No. 2, the method/ mechanism evaluation of spectrum utilization varies on case to case basis depending upon tine incumbent users, services being offered, technology, areas of operation and potential future use of the band under consideration. Similarly, the end objective behind the spectrum utilization evaluation exercise also affects the method/ mechanism being adoptee.

Answer 5:

As explained above in reply to Question No. 2 & 4. The border in the evaluation of spectrum utilization varies on case to case basis depending upon the incumbent users, services being offered, technology, areas of operation and potential future use of the band under consideration. Similarly, the end objective behind the spectrum utilization evaluation also has a bearing on the border being decided.

Answer 6:

Yes, the results of the spectrum evaluation affects spectrum manager decisions. In the presence of detailed spectrum evaluation report, the spectrum manager can make decision supported through solid evidence and justification,

Part 2:

Answer 1:

At; per Telecom Policy 2015, MolT, on recommendations of FAB and PTA, will prepare and publish a rolling spectrum strategy to be published every year that provides a program for the succeeding three years from the date of publication. The Spectrum Strategy envisages to identify for the succeeding period, a plan for existing spectrum allocation audit. The spectrum audit is carried out on case to case basis necessitated through process of spectrum rearming or repurposing exercise

Answer 2:

i:

Spectrum audit can be conducted for any band and or service.

ii:

The spectrum audit is mainly conducted for spectrum occupancy, utilization and associated other technical parameters

iii:

The spectrum audit is conducted by FAB. However, the consultation also involves MoIT&T, P'i A and the incumbent users.

Answer 3:-

### Sri Lanka

Answer 1:

Not specific policies/ regulatory measures about spectrum utilization in Sri Lanka.

Answer 2:

Yes. Especially in IMT services, TRC introduces coverage obligations which need to be fulfilled within certain time period.

Answer 3:

As mentioned in Answer 2, TRC evaluates the achievement of coverage obligations in regular intervals. The operator shall give a surety in terms of bank guarantee for the obligation. If the operator fails to fulfill the obligation, TRC forfeits the bank guarantee as a penalty. If the operator is continuing the failure to meet the obligation the frequency shall be withdrawn.

Answer 4:

TRC requests operators to submit related information and TRC verifies those information in random basis.

Answer 5:

Not yet define

Answer 6:

Results of the evaluation will help to formulate future policies and regulatory frameworks.

Part 2:

Answer 1:

Just started with broadcasting service.

Answer 2:

i:

TRC stars with broadcasting service and we are formulating new rules to do spectrum audit in other services also.

ii:

In broadcasting TRC measures transmitter power, deviation, bandwidth, coverage etc.

iii:

By the regulator

Answer 3:-

### Bangladesh

Answer 1:

There are no specific policies/regulatory measures about spectrum utilization in Bangladesh.

Answer 2:

BTRC has no regulatory initiative to carry out the evaluation of spectrum utilization according to ITU-R recommendation.

Answer 3:

Not Applicable.

Answer 4:

Since BTRC does not carry out the evaluation of spectrum utilization according to ITU-R recommendation, this question is not relevant for Bangladesh.

Answer 5:

Since BTRC does not carry out the evaluation of spectrum utilization according to ITU-R recommendation, this question is not relevant for Bangladesh.

Answer 6:

Since BTRC does not carry out the evaluation of spectrum utilization according to ITU-R recommendation, this question is not relevant for Bangladesh.

Part 2:

Answer 1:

No.

Answer 2:

i:

Not Applicable.

ii:

Not Applicable.

iii:

Not Applicable.

Answer 3:

Not Applicable.

### Nepal

Answer 1:

No

Answer 2:

No

Answer 3:

Not Applicable.

Answer 4:

No.

Answer 5:

No.

Answer 6:

Not Applicable.

Part 2:

Answer 1:

No.

Answer 2:

i:

Not Applicable.

ii:

Not Applicable.

iii:

Not Applicable.

Answer 3:

We have implemented the policy of minimum and maximum spectrum in each of the frequency bands that can be assigned to the service provider. Apart from few bands (namely 900 MHz and 1800 MHz), enough spectrum is available in other bands at present. Due to low demand and associated capping in the bands, we have not prioritized the evaluation of spectrum utilization. However, we are studying the possibilities of spectrum utilization evaluation in our context.

### India

Answer 1:

The National Frequency Allocation Plan is used for planning spectrum utilization in the country.

Answer 2:

Whenever new bands are identiﬁed in the World Radiocommunication Conferences for any radiocommunication services, Government carries out necessary review of the occupancy of bands for making available spectrum for such services.

Answer 3:

Concurrent audit was carried out recently in India in which the spectrum utilization of the major users was audited and the recommendations provided are being evaluated and implemented. The preform for audit is attached herewith.

Answer 4:

Refer responses to question 2 & 3.

Answer 5:

Refer responses to question 2 & 3

Answer 6:

Necessary steps for surrender/vacation by the incumbent users in accordance with Audit is being followed up.

Part 2:

Answer 1:

Please refer response to Part 1 question 3

Answer 2:

i:

Please refer response to Part 1 question 3

ii:

Please refer CAG audit report 2 of 2022 (attached)

iii:

The Performance Audit on ‘Management of Spectrum assigned on the administrative basis to Government Departments/Agencies’ covering the period 2012 to 2021 was conducted during March –July 2021 by the Comptroller and Auditor General of India (CAG). CAG audit report of 2022 is attached.

Answer 3: Not applicable

### Iran

Answer 1:

Yes, in Iran, we are receiving information from MNOs and FWAs continuously for calculating spectrum utilization, also by spectrum monitoring and audit the administration founds about utilization spectrum by licenses.

Answer 2:

The administration accordance with ITU recommendations, prepared a software for calculating spectrum urtication’s MNOs and FWAs, also result of spectrum monitoring is evaluating.

Answer 3:

Base of analyze and evaluate of utilization is based on ITU recommendation SM.1046

Answer 4:

According with answer of Q.3

Answer 5:

The border is defined based on all of result that achieved in 3 years result evaluate as average.

Answer 6:

Yes, the result is effecting in all of spectrum assignment decisions, especially when an operator request additional band by sharing or trading form.

Part 2:

Answer 1:

Yes, as continuously. The administration is carrying out all of spectrum users.

Answer 2:

i:  all of process of audit is carrying out by regional offices which are part of administration’s organization.

ii: Power, gain of antenna, tower’s Height, frequency, status of devices.

iii: according to i.

Answer 3: -

### Bhutan

Answer 1:

We have the Information Communication and Media Act 2018 which also highlights the

Usage of radio frequency spectrum. We also have the National Radio Rules and Regulations 2021 And the National Frequency Allocation Plan 2021.

Answer 2:

Not taken any initiatives so far.

Answer 3:

We have not carried out the spectrum utilization based on per radio service(s). However, we do check the spectrum occupancy once the spectrum has been assigned to the licensees.

Answer 4:

From ITU recommendations and suggestions made by the APT/ STARC working group.

Answer 5:

No as of now.

Answer 6:

N/A

Part 2:

Answer 1:

Not as of now. We only do spectrum occupancy monitoring.

Answer 2:

i:

Not Applicable.

ii:

Not Applicable.

iii:

Not Applicable.

Answer 3:

We have been carrying out the spectrum occupancy monitoring once the spectrum has been assigned

### Afghanistan

Answer 1:

No.

Answer 2:

No.

Answer 3:

Currently there are more than 35000 frequency permit letters assigned to different organization for utilizing the spectrum for different services, based on their request and normally renewed on annual basis. Majority of these permit letters belong to MNOs, ISPs, TTSPs, . . . . it is very important to be mentioned that the information in this paragraph is very general, the spectrum utilization evaluation is not yet in practice.

Answer 4:

No.

Answer 5:

No.

Answer 6:

Spectrum utilization evaluation is not yet in practice

Part 2:

Answer 1:

No.

Answer 2:

i:

-

ii:

-

iii:

-

Answer 3:

Spectrum utilization evaluation is not yet in practice.

### Maldives

Answer 1:

We don’t have guideline for utilization of spectrum.

Answer 2:

We don’t have.

Answer 3:

NA.

Answer 4:

NA.

Answer 5:

NA.

Answer 6:

Not Applicable

Part 2:

Answer 1:

We haven’t start

Answer 2:

i:

NA.

ii:

NA.

iii:

NA.

Answer 3:

Not applicable.

References

- Recommendation  ITU-R  SM.1046-3, Definition of spectrum use and efficiency of a radio system

- Recommendation ITU-R SM.1880-1, Spectrum occupancy measurement and evaluation

- Survey on Spectrum Utilization in Europe: Measurements, Analyses and Observations

- RECOMMENDATION ITU-R SM.1599-1, Determination of the geographical and frequency distribution of the spectrum utilization factor for frequency planning purposes.

- Report ITU-R  SM.2015-2, Methods for determining national long-term strategies for spectrum utilization

- Spectrum Occupancy Measurements and Analysis in Beijing, Jintao Xue, Zhiyong Feng, Ping Zhang.

- ITU Spectrum Monitoring Hand Book, 2015

- Report of the Comptroller and Auditor General of India on Management of Spectrum assigned on the Administrative basis to Government Departments/ Agencies

_

Circuit Switch

Packet Switch